Module 1.

Computer Fundamentals

Er. Tulshidas Mane

Asst. Prof. VPPCOE & VA

Module Outcomes

✔ Convert the given number into the specified number system.

✔ Perform the binary airthmetic operation on the given  binary numbers.

✔ Convert the given coded number into other specified  code.

✔ Add the given two decimal numbers using BCD code. 2

Analog Vs Digital

Analog Signal

• Continuous

• Infinite range of values • More exact values, but  more difficult to work with

Example:

Digital Signal

• Discrete

• Finite range of values (2) • Not as exact as analog,  but easier to work with

A digital thermostat in a room displays a temperature of 72°. An analog  thermometer measures the room temperature at 72.482°. The analog value is continuous and more accurate, but the digital value is more than adequate for the application and significantly easier to process electronically.

3

Example of Analog Vs Digital System

Digital

advantages:

Battery life

Programmability

Accuracy 4

The World is Analog

✔ The world we live in is analog.

✔ We are analog.

✔ Any inputs we can perceive are analog.

✔ For example,

– sounds are analog signals; they are continuous time and continuous value.

– Our ears listen to analog signals and we speak with analog signals. – Images, pictures, and video are all analog at the source and our  eyes are analog sensors.

– Measuring our heartbeat, tracking our activity, all requires  processing analog sensor information.

5

Examples of Analog Signal

✔ An analog signal can be any time-varying signal.

✔ Minimum and maximum values can be either positive or negative. ✔ They can be periodic (repeating) or non-periodic.

✔ Sine waves and square waves are two common analog signals. ✔ Note that this square wave is not a digital signal because its minimum  value is negative.

✔ Video and Audio

0

volts

Sine

Wave

Random Periodic

2 3

Parts of Analog Signal

Period

(T)

Amplitude  (peak-to-peak)

Amplitude (peak)

Frequency:F =1 Hz

T

7

Pros and Cons Analog Signal

⮚ Advantages

✔ Major advantages of the Analog signal is infinite amount of data.

✔ Density is much higher.

✔ Easy processing.

⮚ Disadvantages

✔ Unwanted noise in recording.

✔ If we transmit data at long distance then unwanted  disturbance is there.

✔ Generation loss is also a big con of analog signals. 8

Logic Levels

Before examining digital signals, we must define logic levels. A logic

level is a voltage level that represents a defined digital state. ✔ Logic HIGH: The higher of two voltages, typically 5 volts ✔ Logic LOW: The lower of two voltages, typically 0 volts

5.0 v

2.0 v

0.8 v 0.0 v

Logic High

Invalid

Logic

Level

Logic Low

| Logic Level | Voltage | True/False | On/Off | 0/1 |
| --- | --- | --- | --- | --- |
| HIGH | 5 volts | True | On | 1 |
| LOW | 0 volts | False | Off | 0 |

9

Examples of Digital Signal

✔ Digital signal are commonly referred to as square waves or clock signals.

✔ Their minimum value must be 0 volts, and their maximum value must be 5 volts.

✔ They can be periodic (repeating) or non-periodic.

✔ The time the signal is high (tH) can vary anywhere from 1% of the period to 99% of the period.

✔ Text and Integers.

5 volts

0 volts

10

Parts of Digital Signal

Amplitude:

For digital signals, this will ALWAYSbe 5 volts.

Period:

The time it takes for a periodic signal to repeat. (seconds)

Frequency:

A measure of the number of

occurrences of the signal per second.  (Hertz, Hz)

Time High (tH):

The time the signal is at 5 v.

Time Low (tL):

The time the signal is at 0 v.

AmplitudeTime

High

(tH)

Period (T)

Frequency:

Time  Low  (tL)

Falling Edge Rising Edge

Duty Cycle:

The ratio of tH to the total period (T). Rising Edge:

A 0-to-1 transition of the signal.

Falling Edge:

A 1-to-0 transition of the signal.

F =1 Hz T

DutyCycle =tH ×100%T

11

Pros and Cons Digital Signal

⮚Advantages

✔Because of their digital nature they can travel  faster in over digital lines.

✔Ability to transfer more data as compared to  analog.

⮚Disadvantages

✔Greater bandwidth is essential.

✔Systems and processing is more complex. 12

Example of using digital over analog: Noisy channel Noisy channel

✔ Digital systems are less sensitive to

noise

✔ As long as 0 is distinguishable from 1

13

Analog Vs Digital

⮚Analog system

✔The physical quantities or signals  continuously over a specified range.

⮚Digital system

may vary

✔The physicalquantities or signals can assume only discrete values.

✔Greater accuracy

14

Analog Vs DigitalX(t)

t

Analog signal

X(t)

t

Digital signal

15

Advantages of Digital System over Analog System

✔Digital Systems are easier to design ✔Information storage is easy

✔Accuracy & Precision are greater

✔Digital systems are more versatile

✔Digital circuits are less affected by noise

✔More digital circuitry can be fabricated on IC chips

✔Reliability is more

16

Number System and Codes

17

Number System

✔A number system defines a set of values  used to represent quantity.

18

Different Number Systems

✔ Decimal Number System

- Base/Radix 10

✔ Binary Number System

- Base/Radix 2

✔ Octal Number System

- Base/Radix 8

✔ Hexadecimal Number System

Base/Radix 16

Decimal Number System

✔ Decimal number system contains ten unique symbols

0,1,2,3,4,5,6,7,8 and 9

✔ Since counting in decimal involves ten symbols,  we can say that its base or radix is ten.

✔ It is a positional weighted system

20

Decimal Number System

✔In this system, any number (integer, fraction or mixed) of any magnitude can be represented by the use of these ten symbols only

✔Each symbols in the number is called a “Digit”

21

Decimal Number System

Structure:

MSD LSD

Decimal No. ...... d3

d2 d1 d0 . d − 1 d − 2

....

Positional Weights

103 102 101 100 10−1 10−2

Decimal Point

22

Decimal Number System

✔ MSD: The leftmost digit in any number representation, which has the greatest positional weight out of all the digits present in that number is called the “Most Significant Digit” (MSD)

✔ LSD: The rightmost digit in any number representation, which has the least positional weight out of all the digits present in that number is called the “Least Significant Digit” (LSD)

23

Decimal Number System

⮚Examples

1214

1897

9875.54

24

Binary Number System

✔Binary number system is a positional weighted system

✔It contains two unique symbols 0 and 1 ✔Since counting in binary involves two symbols, we can  say that its base or radix is two.

25

Binary Number System

✔A binary digit is called a “Bit”

✔A binary number consistsof a sequenceof  bits, each of which is either a 0 or a 1. ✔The binary point separatesthe integer and fraction parts

26

Binary Number System Structure:

Binary No.

MSB LSBb2 b1 b0 ...... b3. b − 1 b − 2

....

Positional Weights

23 22 21 202−1 2−2

Binary Point

Binary Number System

✔MSB: The leftmost bit in a given binary number with the highest positional weight is called the “Most Significant Bit” (MSB)

✔LSB: The rightmost bit in a given binary number with the lowest positional weight is called the “Least Significant Bit” (LSB) 28

Binary Number System

| Decimal No. | Binary No. |
| --- | --- |
| 0 | 0000 |
| 1 | 0001 |
| 2 | 0010 |
| 3 | 0011 |
| 4 | 0100 |
| 5 | 0101 |
| 6 | 0110 |
| 7 | 0111 |

| Decimal No. | Binary No. |
| --- | --- |
| 8 | 1000 |
| 9 | 1001 |
| 10 | 1010 |
| 11 | 1011 |
| 12 | 1100 |
| 13 | 1101 |
| 14 | 1110 |
| 15 | 1111 |

29

Terms related to Binary Numbers

✔BIT: The binary digits (0 and 1) are called bits.

- Single unit in binary digit is called “Bit”

- Example 1

0

30

Terms related to Binary Numbers

✔NIBBLE: A nibble is a combination of 4  binary bits.

Examples, 1110

0000

1001

0101

31

Terms related to Binary Numbers

✔ BYTE: A byte is a combination of 8 binary bits.

✔The number of distinct values represented by a byte is  256 ranging from 0000 0000 to 1111 1111.

b7 b6 b5

Higher order nibble

MSB

b4 b3 b2 b1 b0

Lower order

nibble

LSB

32

Terms related to Binary Numbers

✔WORD: A word is a combination of 16 binary bits. Hence it consists of two bytes.

b15 b14 b13 b12 b11 b10 Higher order byte

b9 b8 b7

b6 b5 b4 b3 b2 b1 b0 Lower order byte

MSB LSB

33

Terms related to Binary Numbers

✔DOUBLE WORD: A double word is exactly  what its name implies, two words.

-It is a combination of 32 binary bits. 34

Octal Number System

✔Octal number system  system

is a positional weighted

✔It containseight unique symbols0,1,2,3,4,5,6 and 7

✔Since counting in octal involves eight symbols,  we can say that its base or radix is eight.

35

Octal Number System

✔The largest value of a digit in the octal system will be 7.

✔That means the octal number higher than 7  will not be 8, instead of that it will be 10.

36

Octal Number System Structure:

Octal No.

MSD LSDO2 O1 O0 ...... O3. O− 1 O − 2

....

Positional Weights

80 83 82 81 8−1 8−2

Radix Point37

Octal Number System

✔Since its base 8 = 23 , every 3 bit group of  binary can be represented by an octal digit.

✔An octal number is thus 1/3rd the length  of the corresponding binary number

38

Octal Number System

| Decimal No. | Binary No. | Octal No. |
| --- | --- | --- |
| 0 | 0000 | 0 |
| 1 | 0001 | 1 |
| 2 | 0010 | 2 |
| 3 | 0011 | 3 |
| 4 | 0100 | 4 |
| 5 6 | 0101 0110 | 5 6 |
| 7 | 0111 | 7 |
| 8 | 1000 | 10 |
| 9 | 1001 | 11 |
| 10 | 1010 | 12 |
| 11 | 1011 | 13 |
| 12 | 1100 | 14 |
| 13 | 1101 | 15 |

Hexadecimal Number System (HEX)

✔Binary numbers are long. These numbers are fine for machines but are too lengthy to be handled by human beings. So there is a need to represent the binary numbers concisely.

✔One number system developed with this objective is the hexadecimal number system (or Hex)

Hexadecimal Number System (HEX)

✔Hex number system is a positional weighted  system

✔It contains sixteen unique  0,1,2,3,4,5,6,7,8,9,A,B,C,D,E and F.

symbols

✔Since counting in hex involves sixteen  symbols, we can say that its base or radix is sixteen.

Hexadecimal Number System (HEX)

Structure:

Hex No.

MSD LSD...... H3 H2 H1 H0. H− 1 H − 2

163 162 161 16−1 16−2

....

Positional Weights

160

Radix Point

Hexadecimal Number System (HEX)

✔Since its base16 = 24, every 4 bit group of binary  can be represented by an hex digit.

✔An hex number is thus 1/4th the length of the corresponding binary number

✔The hex system is particularly useful for human communications with computer

Hexadecimal Number System (HEX)

| Decimal No. | Binary No. | Hex No. |
| --- | --- | --- |
| 0 | 0000 | 0 |
| 1 | 0001 | 1 |
| 2 | 0010 | 2 |
| 3 | 0011 | 3 |
| 4 | 0100 | 4 |
| 5 | 0101 | 5 |
| 6 | 0110 | 6 |
| 7 | 0111 | 7 |

| Decimal No. Binary No. |  | Hex No. |
| --- | --- | --- |
| 8 | 1000 | 8 |
| 9 | 1001 | 9 |
| 10 | 1010 | A |
| 11 | 1011 | B |
| 12 | 1100 | C |
| 13 | 1101 | D |
| 14 | 1110 | E |
| 15 | 1111 | F |

Conversion Among Bases

Possibilities

Decimal Octal

Binary

Hexadecimal

Conversion of Decimal number into Binary number (Integer Number)

Procedure:

1. Divide the decimal no by the base 2, noting the remainder.

2. Continue to divide the quotient by 2 until there is nothing left, keeping the track of the remainders from each step.

3. List the remainder values in reverse order to find the number‟s binary equivalent

Example: Convert 105 decimal number in to it’s  equivalent binary number.

Example: Convert 105 decimal number in to it‟s  equivalent binary number.

2 105

Example: Convert 105 decimal number in to it‟s  equivalent binary number.

2 105

2 52 1

Example: Convert 105 decimal number in to it‟s  equivalent binary number.

2 105

2 52 2 26

1 0

Example: Convert 105 decimal number in to it‟s  equivalent binary number.

2 105

2 52 2 26 2 13

1 0 0

Example: Convert 105 decimal number in to it‟s  equivalent binary number.

2 105

2 52 2 26 2 13 2 6

1 0 0 1

Example: Convert 105 decimal number in to it‟s  equivalent binary number.

2 105

2 52 2 26 2 13 2 6 2 3

1 0 0 1 0

Example: Convert 105 decimal number in to it‟s  equivalent binary number.

2 105

2 52 2 26 2 13 2 6

2 3 2 1

1 0 0 1 0 1

Example: Convert 105 decimal number in to it‟s  equivalent binary number.

2 105

2 52 2 26 2 13 2 6

2 3 2 1 0

1 0 0 1 0 1 1

Example: Convert 105 decimal number in to it‟s  equivalent binary number.

2 105

2 52 2 26 2 13 2 6

2 3 2 1 0

1 0 0 1 0 1 1

LSB

MSB

(105)10 = (1101001)2

Conversion of Decimal number into Binary number (Fractional Number)

Procedure:

1. Multiply the given fractional number by base 2. 2. Record the carry generated in this multiplication as MSB.

3. Multiply only the fractional number of the product in  step 2 by 2 and record the carry as the next bit to MSB.

4. Repeat the steps 2 and 3 up to 5 bits. The last carry  will represent the LSB of equivalent binary number

Example: Convert 0.42 decimal number in  to it’s equivalent binary number.

Example: Convert 0.42 decimal number in  to it‟s equivalent binary number.

0.42 X 2 = 0.84 0

Example: Convert 0.42 decimal number in  to it‟s equivalent binary number.

0.42 X 2 = 0.84 0

0.84 X 2 = 1.68 1

Example: Convert 0.42 decimal number in  to it‟s equivalent binary number.

0.42 X 2 = 0.84 0

0.84 X 2 = 1.68 1

0.68 X 2 = 1.36 1

Example: Convert 0.42 decimal number in  to it‟s equivalent binary number.

0.42 X 2 = 0.84 0

0.84 X 2 = 1.68 1

0.68 X 2 = 1.36 1

0.36 X 2 = 0.72 0

Example: Convert 0.42 decimal number in  to it‟s equivalent binary number.

0.42 X 2 = 0.84 0

0.84 X 2 = 1.68 1

0.68 X 2 = 1.36 1

0.36 X 2 = 0.72 0

0.72 X 2 = 1.44 1

Example: Convert 0.42 decimal number in  to it‟s equivalent binary number.

0.42 X 2 = 0.84 0 0.84 X 2 = 1.68 1 0.68 X 2 = 1.36 1 0.36 X 2 = 0.72 0 0.72 X 2 = 1.44 1

MSBLSB

(0.42)10 = (0.01101)2

Exercise

• Convert following Decimal Numbers in to its equivalent Binary Number:

1. (1248.56)10 = ( ? )2

2. (8957.75)10 = ( ? )2

3. (420.6)10 = ( ? )2

4. (8476.47)10 = ( ? )2

Conversion of Decimal Number into Octal Number  (Integer Number)

Procedure:

1. Divide the decimal no by the base 8, noting the remainder.

2. Continue to divide the quotient by 8 until there is nothing left, keeping the track of the remainders from each step.

3. List the remainder values in reverse order to find the number‟s octal equivalent

Example: Convert 204 decimal number in  to it’s equivalent octal number.

Example: Convert 204 decimal number in  to it‟s equivalent octal number.

8 204

Example: Convert 204 decimal number in  to it‟s equivalent octal number.

2 5

8 204

8 25 4

8

204

- 16

44

- 40 4

Example: Convert 204 decimal number in  to it‟s equivalent octal number.

3

8 204 8 25 8 3

8

4

1

25

- 24 1

Example: Convert 204 decimal number in to it‟s  equivalent octal number.

8 204

8 25 8 3 0

4 1 3

Example: Convert 204 decimal number in to it‟s  equivalent octal number.

8 204

LSD

8 25 8 3 0

4

1

3 MSD

(204)10 = (314)8

Conversion of Decimal Number into Octal Number  (Fractional Number)

Procedure:

1. Multiply the given fractional number by base 8. 2. Record the carry generated in this multiplication as MSD.

3. Multiply only the fractional number of the product in  step 2 by 8 and record the carry as the next bit toMSD. 4. Repeat the steps 2 and 3 up to 5 bits. The last carry will  represent the LSD of equivalent octal number

Example: Convert 0.6234 decimal number in to  it’s equivalent Octal number.

Example: Convert 0.6234 decimal number in to it‟s  equivalent Octal number.

0.6234 X 8 = 4.9872 4

Example: Convert 0.6234 decimal number in to it‟s  equivalent Octal number.

0.6234 X 8 = 4.9872 4

0.9872 X 8 = 7.8976 7

Example: Convert 0.6234 decimal number in  to it‟s equivalent Octal number.

0.6234 X 8 = 4.9872 4

0.9872 X 8 = 7.8976 7

0.8976 X 8 = 7.1808 7

Example: Convert 0.6234 decimal number in  to it‟s equivalent Octal number.

0.6234 X 8 = 4.9872 4

0.9872 X 8 = 7.8976 7

0.8976 X 8 = 7.1808 7

0.1808 X 8 = 1.4464 1

Example: Convert 0.6234 decimal number in  to it‟s equivalent Octal number.

0.6234 X 8 = 4.9872 4

0.9872 X 8 = 7.8976 7

0.8976 X 8 = 7.1808 7

0.1808 X 8 = 1.4464 1

0.4464 X 8 = 3.5712 3

Example: Convert 0.6234 decimal number in to it‟s  equivalent Octal number.

0.6234 X 8 = 4.9872 4 0.9872 X 8 = 7.8976 7 0.8976 X 8 = 7.1808 7 0.1808 X 8 = 1.4464 1

MSD

0.4464 X 8 = 3.5712 3 LSD (0.6234)10 = (0.47713)8